Lower Ram All Electric

Clampup pneumatic adjustable 0-1200 lbf.  Clear opening is 150mm with lower tool in place.

Load cell to control and measure upset

Roller screw servoed 18 kip max, stroke 180mm, can install 5/16 dia 2017 rivets and ¼ dia 7050 rivets

Lower tool two 16mm bolt attachment, becomes solid for bolt insertion, manual rotation

Upper Ram All Electric

Matching roller screw to lower for bolt insertion up to 5/8, stroke 150mm

Ram rotates 30 degrees to accept the rivet

Electroimpact patented air gap sensor to prevent panel damage from laid down rivets

Upper riveting tool attaches with Schunk Vero.  Pushbutton release.

Upper pressure foot bushing

Moving upper pressure plate 10mm stroke

Bushing height 50mm

Manual rotation

Spindle All Electric

Drill spindle is Fischer SD60124 with pneumatic drawbar for HSK40.

Spindle provides 10 Ntm continuous torque to HSK40 tooling.  Can drill 3/4” in aluminum.

Spindle is liquid cooled.

Max RPM 18000

Spindle stroke is 110mm

Boelube headstone spray lube

Chip blower

C-Frame

Throat Depth 1.1m.  Will be adjusted to meet your requirements.

Controls

Siemens PLC

Power 400VAC 3ph

Unique and Flexible Hopper Feed System

Fingers rotates to a 30 degree angle to match the hopper ramp angle.

Minimum grip rivet that can be fed is 4.5 (0.28”).  Fastener feed is not impeded by reverse shingling due to the in-line injection feature.

Maximum length of bolt that can be fed is two inch pintail.

Hopper changer allows six different hoppers to be mounted and manually or auto selected.

See the wide variety of fasteners that can be automatically fed and installed below.

Positioning

Laseraim laser dot

Push pedal to insert fastener

Chip Vacuum

Dry vacuum

Shutoff valve to allow Boelube spray

Cost

Cost to purchase is $500,000 complete with tooling

Less than the cost to refurb an old-fashioned hydraulic riveter.

Lease to own